Resetting the BIOS (7685T-8X)

- Attach a USB keyboard and mouse to the ePro unit.

- Attach an External Monitor to the ePro unit.

- Press the Delete Key, and then apply power. Release the Delete Key when Bios menu appears on the monitor.

- Arrow to Advanced BIOS Features and press the Enter key.

- Arrow to First Boot Device and press the Enter key.

- Arrow to Hard Disk and press the Enter key.

- Press Escape Key

- Arrow down to Advanced Chipset Features and press the Enter Key.

- Arrow to AGP & P2P Bridge Control and press the Enter Key.

- Arrow to VGA Share Memory Size and use the + key to select 32 M.

- Arrow to Select Display Device and use the + key to select CRT+LCD.

- Arrow to Panel Type and use the + key to select 800X600 1Ch 18bit.

- Press ESC twice to escape back to the Bios main menu.

- Arrow to Integrated Peripherals and press the Enter Key.

- Arrow to VIA On Chip IDE Device and press the Enter key.

- Arrow to SATA Controller and use the + key to set it to Disable.

- Arrow to IDE HDD Block Mode and use the + key to set it to Disable.

- Press the F10 key on keyboard, enter Y (yes) then press the Enter key. This will save the settings.